Implementation Document
Semantic 3D Urban Digital Twin Pipeline

Target stack: Python, CloudCompare, C++/Qt, Unity

Purpose: define a production-style pipeline for generating terrain, buildings, roads, bridges, and semantic metadata from Swedish geospatial datasets, then rendering them as separately selectable interactive assets in Unity.

Primary source roles: Laserdata NH for accurate terrain and structural heights; Ytmodell for dense visible-surface detail; Byggnad for building footprints; transport and hydrography layers for roads and bridge logic; population and address layers for semantics.

# 1. System overview

The system is organized as a multi-layer pipeline. Geometry is generated from authoritative vectors plus measured heights. Semantics are joined separately. Runtime rendering and interaction are handled in Unity rather than in the preprocessing layer.

| Layer | Main tools | Output | Responsibility |
| --- | --- | --- | --- |
| Source data | Raw datasets | LAZ / GPKG / SHP / TIF | Authoritative measurement and GIS inputs |
| Processing | Python | Meshes, heightmaps, metadata | ETL, reconstruction, semantic joins |
| Validation | CloudCompare, Qt | QC reports, manual fixes | Inspection and quality control |
| Runtime | Unity | Interactive 3D city | Rendering, hover/click, filtering |

# 2. Source datasets and intended use

Use point clouds as measurement sources, not as the final runtime geometry. Use vectors as structured city objects, and export final assets as meshes or heightmaps.

| Dataset | Type | Primary use | Priority |
| --- | --- | --- | --- |
| Byggnad | Vector building footprints | Building boundaries and stable IDs | Essential |
| Laserdata NH | Classified LiDAR point cloud | Terrain, building heights, bridge evidence | Essential |
| Ytmodell | Photogrammetric surface point cloud | Surface refinement and visible roof detail | Important |
| Transport networks / Vägkartan | Vector lines | Road mesh generation | Essential |
| Hydrography | Vector polygons/lines | Bridge crossing validation | Important |
| Population / address / POI | Thematic and tabular | Building semantics and hover data | Essential |

Rule: buildings should not be reconstructed directly from raw point clouds when authoritative footprints are available. Use footprints plus LiDAR-derived heights for clean, scalable geometry.

# 3. Project structure and data contracts

A predictable folder structure is required for industrial-scale processing, QA, and reruns. Keep raw data immutable. Store intermediate products and final runtime outputs separately.

project_root/

data_raw/          # authoritative inputs

data_intermediate/ # normalized layers, tile caches, height tables

data_output/       # exported runtime assets by tile

tools/             # python scripts, Qt tools, import helpers

unity_project/     # runtime application

Every exported runtime object must have a stable ID. IDs must not depend on mesh order, Unity hierarchy order, or ad hoc manual naming.

# 4. Processing pipeline

## 4.1 Ingestion and normalization

Read all raw datasets, validate CRS, normalize attributes, repair invalid geometries, and assign every source object to a processing tile. Internally standardize everything in SWEREF 99 TM with metric heights.

## 4.2 Terrain extraction

Filter LiDAR ground points and interpolate a terrain surface. Export either a terrain mesh or a Unity-compatible heightmap. Use terrain rasters only as fallback or validation layers.

## 4.3 Building height extraction

For each building footprint, estimate local ground elevation outside or around the footprint and roof elevation from LiDAR or DSM points inside the footprint. Store height, roof elevation, and QC values per building.

## 4.4 Building mesh generation

Generate LOD1 by extruding each footprint from ground elevation to roof elevation. Keep one exported mesh per building so each object remains separately selectable and independently colorable at runtime.

## 4.5 Road mesh generation

Buffer road centerlines using width-by-class rules, fit the resulting polygons to terrain, triangulate, and export road meshes per segment or tile.

## 4.6 Bridge detection and bridge meshes

Detect candidate bridges where roads cross water or terrain depressions and the road surface is above local terrain by a threshold. Use LiDAR bridge-class evidence where available. Export each bridge as its own asset.

## 4.7 Semantic enrichment

Join addresses, POIs, land-use or institutional datasets, and population sources to building IDs. Normalize raw labels into a controlled building taxonomy such as residential_house, school, hospital, office, industrial, retail, government, or unknown.

## 4.8 Export

Export geometry as GLB/GLTF and store metadata in SQLite plus optional per-tile JSON caches. Each tile should be independently loadable in Unity.

# 5. Semantic schema

The metadata layer should be designed as a first-class system. Geometry lives in mesh files; truth about the object lives in metadata.

| Entity | Required fields | Notes |
| --- | --- | --- |
| Building | id, tile_id, type_normalized, address, population_estimate, height_m, mesh_file | One logical building object; separate runtime selection |
| Road segment | id, tile_id, class_normalized, width_m, mesh_file | Road mesh generated from vectors |
| Bridge | id, parent_road_id, deck_elevation_m, span_length_m, mesh_file | Separate asset and category |
| Tile | tile_id, local_origin, bounds, asset_list | Runtime streaming unit |

Recommended production storage: SQLite for metadata, GLB for geometry, optional JSON for debugging and inspection.

# 6. Classification and color policy

Classification is primarily a semantic join problem, not a geometry problem. The preferred order is official building-use data, then attributes already present in building data, then municipal or OSM POIs, then rule-based inference, and finally unknown.

| Normalized class | Default runtime color |
| --- | --- |
| residential_house / residential_apartment | yellow / orange |
| school | blue |
| hospital | red |
| office | cyan |
| industrial | gray |
| retail | green |
| government | purple |
| unknown | white |

# 7. Unity runtime implementation

Use tile-based loading. Each tile contains terrain, buildings, roads, bridges, and a metadata subset. Load only nearby tiles and unload distant tiles to maintain frame rate.

Each runtime object must carry a stable semantic ID. On hover or click, Unity should raycast to the object, read its ID, query the metadata dictionary or SQLite-backed cache, and populate the UI panel.

Support multiple visual modes: category colors, height colors, population colors, and highlight/filter modes such as schools only or hospitals only.

Keep logical object separation even if rendering optimization later introduces batching or instancing. Selection must still resolve back to one building ID.

# 8. Development milestones

Milestone 1 — vertical slice: One small urban area with terrain, 50–200 buildings, roads, one bridge, hover metadata, and category colors.

Milestone 2 — semantic expansion: Address integration, population estimates, and normalized building classification.

Milestone 3 — performance and scaling: Tile streaming, LODs, asynchronous loading, profiling, and QA tooling.

Milestone 4 — higher detail: Optional roof refinement, better bridge geometry, and richer UI/query modes.

# 9. Risks, quality gates, and non-goals

Do not use raw point clouds as final runtime building geometry when clean footprints are available.

Do not export one giant city mesh; preserve tile-based structure.

Do not hardcode all semantics directly inside Unity prefabs; keep metadata external and versioned.

Do not rely on manual CloudCompare steps for the full production pipeline; use it for inspection and QA only.

Before accepting a tile, validate geometry continuity, negative heights, floating buildings, missing metadata, and bridge anomalies.

# 10. Recommended division of responsibilities

| Tool | Primary responsibility |
| --- | --- |
| Python | Main ETL pipeline, mesh generation, joins, export |
| CloudCompare | Inspection, point-cloud QA, geometry sanity checks |
| C++ / Qt | Tile browser, metadata correction tools, QA/editor utilities |
| Unity | Runtime rendering, interaction, UI, streaming, filtering |

# 11. Final implementation rule

Treat LiDAR and DSM as measurement inputs, not as the final city model. Build structured meshes and external semantics on top, then stream those assets into Unity as separately selectable urban objects.